UNOPS-EP-2016-G-003

Purchase of carpentry equipment in support of women entrepreneurship

Nabavka stolarske opreme u okviru programa podrške ženskom preduzetništvu

TECHNICAL SPECIFICATIONS:

| UNOPS minimum technical requirements / UNOPS minimum tehnickih zahteva |  |
| --- | --- |
| LOT 1- Universal" Combination Machine / Kombinovana stolarska mašina |  |
| 1. | "Universal" Combination Machine1/ Kombinovana stolarska mašina 1 CIRCULAR SAW: Engine power S1 (100%) / S6 4,0kW / 5,6 kW Max. saw blade diameter 315mm Rotation speed main blade 4000 rpm Max. cutting height 90° / 45° 103mm/72 mm Scoring unit saw blade Ø 120mm Rotation speed scoring blade 8000rpm Engine power scoring unit S1 (100%) / S6 0,75kW / 1,1kW Max. cutting width at parallel fence 500/1000mm Parallel fence length 1100mm Sliding table 2300mm x 350mm Max. cutting length 2200mm Length sliding table 2544mm Mitre fence length 1940mm - 3480mm SPINDLE SHAPER: Engine power S1 (100%) / S6 5.5 kW / 7,7 kW Spindle diameter 30mm Spindle speed (6 steps) 3000 / 4000 / 5000 / 6000 / 8000 / 10 000 rpm Spindle travel 175mm Max. tool diameter profile cutter/spring mill 250mm Ø table opening 270mm Max. tool diameter below table 250mm Max. tool diameter above table 255mm Mitre fence length (left / right) 345mm x 155mm PLANER: Engine power S1 (100%) / S6 4,0 kW/5,4kW Rotation speed 4500rpm Shaft diameter 100mm Pieces of knives 4 Knife dimension 410mm x 35mm x 3mm / HSS Sliding table length 1800mm Effective planning width 410mm Max. cutting depth 5mm Planning fence (90°-45° tiltable) 1100mm x 150mm THICKNESSER: Table length 700mm Engine power S1 (100%) / S6 4,0kW/5,6kW Thicknessing width 406mm Max. work piece height 225mm Max. cutting depth 4mm Feeding speed 8m / min General characteristics Table height 850 mm diameter suction pipe planing units 120 mm Operating voltage 400V FORMATIZER snaga motora S1 (100%) / S6 4,0kW / 5,6 kW max. prečnik testere 315 mm brzina obrtanja testere 4000 rpm max. visina rezanja 90° / 45° 103 / 72 mm predrezač Ø 120 mm brzina obrtanja predrezača 8000 rpm snaga motora predrezača S1 (100%) / S6 0,75kW / 1,1kW max. širina reza 500 / 1000 mm desni graničnik 1100mm alu. kolica 2300×350 mm max. dužina reza 2200 mm dužina alu. kolica 2544mm levi graničnik 1940-3480mm FREZER snaga motora S1 (100%) / S6 5.5 kW / 7,7 kW prečnik osovine 30mm brzina obrtaja (6 brzina) 3000 / 4000 / 5000 / 6000 / 8000 / 10 000 rpm pomeranje osovine 175mm max. prečnik alata 250 mm Ø otvor na ploči 270mm max. prečnik alata ispod stola 250mm max. prečnik alata iznad stola 255mm graničnik (levo / desno) 345x155mm ABRIHTER snaga motora S1 (100%) / S6 4,0 kW/5,4kW brzina obrtaja 4500 rpm prečnik glave 100 mm broj noževa 4 dimenzije noževa 410x35x3 mm / HSS dužina stola 1800 mm efektivna radna širina 410mm max. dubina skidanja 5 mm graničnik (90°-45° oboriv) 1100x150mm DIHT dužina stola 700 mm snaga motora S1 (100%) / S6 4,0kW/5,6kW radna širina 406mm max. visina RK 225 mm max. visina skidanja 4 mm brzina prolaza 8 m/min Osnovne karakteristike Visina radnog stola 850 mm Precnik usisne cevi 120 mm Radni napon 400V (EQUAL OR SIMILAR TO HOLZMANN K5-VFP-2544) (ISTO ILI SLICNO MODELU HOLZMANN K-5-VFP-2544) |
| 2. | Universal" Combination Machine 2/ Kombinovana stolarska mašina 2 COMBINED 5-OPERATION WOODWORKING MACHINE: - SURFACE TRIMMER (PLANER) - THICKNESS TRIMMER (THICKNESSER) - SLIDING TABLE SAW (CIRCULAR SAW) - MILLING MACHINE (SPINDLE SHAPER) - MORTISING UNIT Grid: 230v Power: 1.5kW Table Length: 960mm Surface Trimmer (PLANER) - Pieces of knives: 2 - Shaft Diameter: ø 72mm - Rotation Speed: 4000 rpm - Knife Width: 250mm - Max. Cutting Depth: 3mm Thickness Trimmer (THICKNESSER) - Max. Work Piece Width: 250mm - Work Piece Thickness: Min: 60mm/Max: 120mm - Min. Work Piece Lentgh: 150mm - Feeding Speed: 4.5m/min Sliding Table Saw (CIRCULAR SAW) - Blade Dimensions: ø 250/ ø 30x3.2mm - Blade Rotation Speed: 3500 rpm - Max. Cutting Thickness: 70mm - Max. Cutting Width (perpendicular): 300mm Milling Machine (SPINDLE SHAPER) - Spindle Diameter: ø 20mm - Max. Milling Width: 40mm - Milling Thickness: 6-60mm Mortising Unit - Max.Tool Grip: ø 12mm - Max. Mortising Depth: 90mm KOMBINOVANA STOLARSKA MAŠINA ZA OBRADU DRVETA SA 5 OPERACIJA: -RENDISANJE POVRŠINSKO (ABRIHTER) -RENDISANJE PO DEBLJINI (DIHT) -KRUŽNA TESTERA (CIRKULAR) -GLODALICA (FREZA) -HORIZONTALNI BUŠAČ Mreža:230v Snaga:1.5kW Dužina radnog stola:960mm Rende površinsko: -Broj noževa:2 -Prečnik osovine noža:fi72mm -Br.obrtaja osovine:4000 o/min -Širina noža:250mm -Max.dubina zahvata noža:3mm Rende po debljini -Max.širina materijala:250mm -Debljine materijala: Min:60mm/Max:120mm -Min.dužina materijala:150mm -Brzina "hranjenja":4.5m/min Kružna testera -Dimenzije testere:fi250/fi30x3.2mm -Br.obrtaja testere:3500 o/min -Max.debljina sečenja:70mm -Max.širina sečenja (poprečno):300mm Glodalica -Prečnik osovone:fi20mm -Max.širina glodanja:40mm -Debljine glodanja:6-60mm Horizontalni bušač -Max.prihvat alata:fi12mm -Max.dubina bušenja:90mm (EQUAL OR SIMILAR TO WOMAX RWM 392 F1) (ISTO ILI SLICNO MODELU WOMAX RWM 392 F1) |

| #### LOT 2- Automatska kant mašina/ Automatic Edge bender |  |
| --- | --- |
| 1. | Automatska kant mašina/ Automatic Edge bender STANDARD EQUIPMENT Electrical cabinet with PLC, pad lockable main switch and overload protection. Emergency pushbuttons. TECHNICAL DATA Worktable dimensions 2600x530 mm Working table height 904 mm Coil edging thickness 0,4-3 mm Strip edging thickness up to 5 mm Min. panel width (coil edging) 65 mm Min. panel length (coil edging) 190 mm Min. panel length (front end trimming) 120 mm Min/max working thickness with edge scraping unit 16/50 mm Feed speed 7 m/min WIDIA tools(trimming)diameter ø75 Z4 End trimming blade ø 90 Z 20 Pneumatic pressure 6,5 bar Suction hood diam. ø 120 mm Suction hood for smoke diam.ø 60 mm Working temperature 20 - 190°C Glue pot unit Motor power 1,2 kW Glue capacity approx. 0,5 Kg End trimming unit Motor power 0,37 kW Blade rotation speed 12.000 rpm Trimming unit Motor power 0,75 kW Cutter rotation speed 12.000 rpm Brushing unit Motor power (each) 0,14 kW Rotation speed 2800 rpm Standardna opremljenost: Elektricno postrojenje sa PLC-om generalnim prekidacem sa zakljucavanjem I termickim relejima Sigurnosni prekidac Tehnicki podaci Dimenzija radnog stola 2600X530 mm Visina radnog stola 904 mm Debljina trake u koturu 0,4-3 mm Debljina letvica do 5 mm Min. širina ploče-obratka (trake u koturu) 65 mm Min. dužina ploče-obratka (trake u koturu 190 mm Min. dužina ploče (prednja pozicija) 120 mm Radna visina agregata sa nožićima u radijusu 16-50 mm Brzina vuče 7 m/min Glodala WIDIA precnika ø 75 Z4 Testera za odsecanje ø 90, Z 20 Potreban radni pritisak 6,5 bara Kapa za odsis ø 120 mm Kapa za aspiraciju gasova ø 60 mm Radna temperatura 20-190oC Agregat posude za lepak: Snaga motora 1,2 kW Kapacitet lepila cca 0,5 kg Agregat za odsecanje: Snaga motora 0,37 kW Brzina okretanja testere 12.000 okr/min Agregat glodala: Snaga motora 0,75 kW Brzina okretanja glodala 12.000 okr/min Agregat četki Snaga motora 0,14 kW Brzina okretanja testere 2.800 okr/min (Equal or similar to CASADEI ALA 20S PLUS) (ISTO ILI SLICNO MODELU CASADEI ALA 20S PLUS) |

| #### LOT3- Tools / Alati |  |
| --- | --- |
| 1. | Cordless Drill Driver / Akumulatorski odvrtač Battery Power 10.8V/1.3 Ah Battery Type Li-Ion Capacity Steel 10 mm Capacity Wood 21 mm Charge Time 50 minute Chuck Capacity 0.8-10mm Power Drill Gearbox Speed 2 No. Torque Setting 16 In the suitcase with extra batteries and a charger Jačina baterije 10.8V/1.3Ah Vrsta baterije Li-Ion Učinak bušenja , čelik 10 mm Učinak bušenja, Drvo 21 mm, Vreme punjenja 50min Stezna glava 0.8-10mm Broj brzina 2 Podešavanje obrtnog momenta u 16 pozicija U koferu sa rezervnim akumulatorom i punjačem (EQUAL OR SIMILAR TO MAKITA DF330DWE) (ISTO ILI SLICNO MODELU MAKITA DF330DWE) |
| 2. | Rotary Drill / Elektronska Bušilica Continuous rating Input minimum 450W Capacity Steel 10 mm Capacity Wood 25 mm Rated speed 0-3400 /min Power supply cord 2.0 m Nominalna snaga min 450 W Učinak bušenja , čelik 10 mm Učinak bušenja, Drvo 25 mm, Br.obrtaja u praznom hodu 0–3400/min Duzina kabla za napajanje 2.0 m (Equal or similar to MAKITA 6413) #### (ISTO ILI SLICNO MODELU MAKITA 6413) |
| 3. | Belt Sander / Tračna brusilica Power Output:720 W No Load Speed:250/ min Belt Dimensions: 75mm x 533mm Sanding Area: 75mm x 190mm Paper Size: 75mm x 533mm Cyclonic action dust canister for a cleaner work environment Voltage power from the network Snaga 720 W Broj obrtaja u praznom hodu 250/m Dimenzija kaiša: 75mm x 533mm Brusna površina 75x190 mm; Dimenzija Traka 75x533 mm; Hepa filter za čistiji radni prostor Napajanje električno iz mreže (EQUAL OR SIMILAR TO BLACK AND DECKER KA 88) #### (ISTO ILI SLICNO MODELU BLACK AND DECKER KA 88) |
| 4. | Barrel Grip Jig Saw / Električna oscilujuća ubodna testera Max Depth of Cut Steel 10mm Max Depth of Cut Wood 135mm Power 720W Stroke Rate 800 – 2800/min Length of stroke 26mm Voltage power from the network Učinak sečenja u čeliku i aluminijumu: 10 mm Učinak sečenja u drvetu: 135 mm Snaga 720W Brzina 800-2800/min Visina hodova: 26 mm Napajanje električno iz mreže (EQUAL OR SIMILAR TO MAKITA 4351 FCT) #### (ISTO ILI SLICNO MODELU MAKITA 4351 FCT) |
| 5 | Plunge Router/ Elektronska glodalica Wattage: 1100W Collet capacity 8 mm No Load Speed: 8,000-24,000rpm Max. Plunge Capacity: 0- 57mm Dust nozzle Electronic speed controller for constant speed under load Kontinuirana ulazna snaga 1100 W Profesionalni prihvat od 8 mm Broj obrtaja 8.000 – 24.000 min-1 Maks. dubina ukopavanja 0-57 mm Mlaznica za prašinu Konstantna kontrola brzine (EQUAL OR SIMILAR TO MAKITA RP1110C) (ISTO ILI SLICNO MODELU MAKITA RP1110C) |
| 6. | Random Orbit Sander / Elektronska Bušilica Wattage: 300 W Orbits per Minute: 4.000 - 12.000 opm Abrasive Disc 123mm Voltage power from the network Snaga 300W Broj obrtaja u minuti 4,000-12,000/min Brusna ploča 123 mm; Napajanje Električno (iz mreže) (Equal or similar to MAKITA BO5041) (ISTO ILI SLICNO MODELU MAKITA BO5041) |
| 7. | Air Compressor/ Vazdušni kompresor High flow rate of 290 liters Pressure 8 bar Power 2.2 kW 3.0 KS Voltage 230 V Protok vazduha 290 l/min Pritisak 8 bara Snaga 2.2 kW 3.0 KS Napon 230 V (EQUAL OR SIMILAR TO VILLAGER VAT LE 100L) #### (ISTO ILI SLICNO MODELU VILLAGER VAT LE 100L) |
| 8. | Jig Saw / Ubodna testera Max Depth of Cut Steel 6mm Max Depth of Cut Wood 65mm Power 450 W Stroke Rate 3100/min Length of Stroke 18mm Voltage power from the network Učinak sečenja u čeliku i aluminijumu: 6 mm Učinak sečenja u drvetu: 65 mm Snaga 450 W Brzina 3100/min Mač Visina hodova: 18 mm Napajanje elektricno iz mreze (Equal or similar to MAKITA 4326) #### (ISTO ILI SLICNO MODELU MAKITA 4326) |
| 9. | Bench drill / Stubna bušilica Power: 700 W Spindle speed: 210-2220/min Orientation drilling table: -45 ° to + 45 ° Drill diameter: Ø 3 mm -16 Projection: 169 mm Max. Drilling depth: 80 mm Orientation drilling table: -45 ° to + 45 ° Snaga 700 W Broj obrtaja u praznom hodu 210-2220/min Stezna glava Ø 3-16 mm Grlo 169 mm Maks. dubina bušenja: 80 mm Zakretanje radnog stola: -45° do +45° (Equal or similar to Einhell SB 1020/1D) #### (ISTO ILI SLICNO MODELU Einhell SB 1020/1D) |