Section IV: Returnable Bidding Forms

Note to Bidders: Instructions to complete each Form are highlighted in blue in each Form.  Please complete the Returnable Biding Forms as instructed and return them as part of your quotation.

The following returnable forms are part of this RFQ and must be completed and returned by bidders as part of their Quotation.

Form A: Quotation submission form

Bidders are requested to complete this form, sign it and return it as part of their bid submission. The bidder shall fill in this form in accordance with the instructions indicated. No alterations to its format shall be permitted and no substitutions shall be accepted.

Date: [Insert submission date]____________________________

Subject: Quotation for the supply of [Insert a brief description of goods/services] __________________________________in ________________________________[Name of country/city], RFQ Case No.[insert RFQ ref number]_______________________________, dated [insert date]_____________________.

We, the undersigned, declare that:

  - We offer to supply in conformity with the bidding documents, including the UNOPS General Conditions of Contract.

  - Our quotation shall be valid for the period of time of [insert number of days which shall not be less than the specified in Section I: Bid Particulars, Period of Validity of Bids] _________________from the date fixed for the bid submission deadline as set out in the RFQ, and it shall remain binding upon us and may be accepted at any time before the expiration of that period;

  - We have no conflict of interest in any activity that would put it, if selected for this assignment, in a conflict of interest with UNOPS;

  - Our firm confirms that the offeror and sub-contractors have not been associated, or had been involved in any way, directly or indirectly, with the preparation of the design, terms of references and/or other documents used as a part of this solicitation;

  - Our firm, its affiliates or subsidiaries—including any subcontractors or suppliers for any part of the Contract—has not been declared ineligible by UNOPS, nor is included in the suspended/ineligibility list of the UN/PD, other UN Agencies, the UN Security Council, and the World Bank, in accordance with Instructions to Bidders Article 3, Eligibility;

  - We embrace the UN Supplier Code of Conduct and adhere to the principles of the UN Global Compact

  - We have not declared bankruptcy, are not involved in bankruptcy or receivership proceedings, and there is no judgment or pending legal action against them that could impair their operations in the foreseeable future.

  - We have not offered and will not offer fees, gifts and/or favours of kind in exchange for this RFQ and will not engage in any such activity during the performance of any Contract awarded.

I, the undersigned, certify that I am duly authorized by [insert full name of bidder] ________________________________to sign this quotation and bind [insert full name of bidder] should UNOPS accept this quotation:

Name: [complete]_______________________________________________________

Title: [complete]_________________________________________________________

Signature: _____________________________________________________________

Provide the name and contact information for the primary contact from your company for this quotation:

Name: [complete]________________________________________________________

Title: [complete]__________________________________________________________

Email address: [complete]____________________________________________________

Telephone: [complete]_______________________________________________________

Form B: Price Schedule Form

Bidders shall fill in this Price Schedule Form in accordance with the instructions indicated.

RFQ reference no: [insert RFQ reference No.]

| Currency | RSD |
| --- | --- |

| Item No | Description of Goods | Quantity | Unit price DAP RSD | Total price DAP RSD |
| --- | --- | --- | --- | --- |
| 1. | Purchase of tool carriage with minimum 211 parts | 10 |  |  |
| TOTAL: |  |  |  |  |

NOTES:

- Transportation, offload, of the tool carriages to the delivery place must be included in the price.

- The offered prices are net of any direct taxes, customs duties and indirect taxes and VAT.

- DAP and supplier to cover the costs of custom clearance agency if needed

Payment terms 30 days accepted: ☐ Yes

- List of subcontractors or suppliers

Bidder must identify the names of all subcontractors/suppliers who will be providing good/services under this Contract and the type of work being subcontracted, if applicable.

[Full legal name and address of subcontractors]

- _________________________________________________

- _________________________________________________

- _________________________________________________

- List of legal entities associated to the Bidder

Bidder must also identify and disclose any information regarding all associated legal entity/s, if any, by providing full legal name and address of the associated entity/s.

[Full legal name and address of the associated legal entity]

- _________________________________________________

- _________________________________________________

- _________________________________________________

Should there be no related entities, the Bidder must sign the following statement to that effect:

[Delete or cross out if not applicable]:

I, the undersigned, certify that there are no legal entities associated to the [insert full name of Bidder]

____________________________________________________. [

- Authorization to sign quotation

I, the undersigned, certify that I am duly authorized by [insert full name of Bidder]

____________________________________________________ to sign this quotation and bind

____________________________________________________[insert full name of Bidder] should UNOPS accept this quotation:

Name	: _____________________________________________________________

Title	: _____________________________________________________________

Date	: _____________________________________________________________

Signature	: _____________________________________________________________

Form C: Technical Quotation Form

RFQ reference no: [insert RFQ reference No.]______________________________________

Name of Bidder: [insert name of Bidder]___________________________________________

Bidders are required to complete the Comparative Data Tables included in Section III: Schedule of Requirements to demonstrate compliance with UNOPS requirements and insert them below. Bidders are NOT allowed to make any change in the “UNOPS requirements” columns of the Comparative Data Tables. Such changes might disqualify your quotation.

Technical specifications for goods – Comparative Data Table

| Item | UNOPS minimum technical requirements | Quantity | Is quotation compliant? Bidder to complete | Details of goods offered. Bidder to complete Insert details of goods offered, including specifications and brand/model offered if applicable |
| --- | --- | --- | --- | --- |
| 1. | Purchase of tool carriage with minimum 211 parts Tool carriage with minimum 211 parts Tool carriage description: - Locking System - Four wheels - Dimensions of tool carriage 588-910 X 360-560 x 850-1050 - The antislip synthetic covering protect drawers and tools from damage - 6 to 8 Drawers - Material: sheet metal Tool carriage must be consisted with the following tools: Set of wrenches in tool tray No. Item; Item; Dimensions of tools. 12; Open end wrench • material: chrome vanadium • surface finish: chrome plated • polished head; 6 x 7 approx. length 120 mm , 8 x 9 approx. length 140mm, 10 x 11 approx. length 160mm, 12 x 13 approx. length 170 mm, 14 x 15 approx. length 192mm, 16 x 17 approx. length 205mm, 18 x 19 approx. length 223mm, 20 x 22 approx. length 232mm, 21 x 23 approx. length 250mm, 24 x 27 approx. length 265mm, 25 x 28 approx. length 280mm, 30 x 32 approx. length 309 mm. 18; Combination wrench, long type • material: chrome vanadium • surface finish: chrome plated • polished head; 6 approx. length 120 mm, 7 approx. length 130mm, 8 approx. length 141mm, 9 approx. length 152mm, 10 approx. length 163mm, 11 approx. length 176mm, 13 approx. length 200mm, 14 approx. length 210mm, 15 approx. length 222mm, 16 approx. length 235mm, 17 approx. length 245mm, 18 approx. length257mm, 19 approx. length 268mm, 20 approx. length280mm, 21 approx. length 291mm, 22 approx. length 303mm,24 approx. length330mm, 27 approx. length 360mm. 8; Offset ring wrench • material: chrome vanadium • surface finish: chrome plated; 6 x 7 approx. length 178mm, 8 x 9 approx. length 190mm, 10 x 11 approx. length 205mm, 12 x 13 approx. length 220mm, 14 x 15 approx. length.235mm, 16 x 17 approx. length 258mm, 18 x 19 approx. length 270mm,20 x 22 approx. length 295mm. 6; Offset open ring wrench • material: chrome vanadium • surface finish: chrome plated • profile up to dimension 12x14 is 6 point, from 17x19 to 30x32 is 12 point; 8 x 10 approx. length 145mm, 10 x 11 approx. length 152mm, 11 x 13 approx. length 165 mm, 12 x 14 approx. length 178mm, 17 x 19 approx. length 210mm, 19 x 22 approx. length 233mm. 9; Ball end hexagon wrench • material: chrome vanadium; 9 (1.5, 2, 2.5, 3, 4, 5, 6, 8, 10). 8; Wrench with TX profile and hole - material: chrome vanadium • surface finish: blacken; 8(TR 9, TR 10, TR 15, TR 20, TR 25, TR 27, TR 30, TR 40). Set of socket wrenches with accessories in tool tray No. Item; Item; Dimensions of tools. 12; Socket 1/4" • material: chrome vanadium • surface finish: chrome plated • drop forged; 4, 4.5, 5, 5.5, 6, 7, 8, 9, 10, 11, 12, 13. 7; Socket with internal TX profile 1/4" • material: chrome vanadium • surface finish: chrome plated • drop forged; E 4, E 5, E 6, E 7, E 8, E 10, E 11. 5; Hexagonal screwdriver socket 1/4" • material: socket: chrome vanadium, chrome plated • material: bit: special tool steel; 3, 4, 5, 6, 7. 8; Screwdriver socket with TX profile 1/4" • material: socket: chrome vanadium, chrome plated • material: bit: special tool steel; TX 8, TX 10, TX 15, TX 20, TX 25, TX 27, TX 30, TX 40. 3; Slotted screwdriver socket 1/4" • material: socket: chrome vanadium, chrome plated • material: bit: special tool steel; 0.8 x 4, 1 x 5.5, 1.2 x 7. 3; Crosstip screwdriver socket 1/4" • material: socket: chrome vanadium, chrome plated • material: bit: special tool steel; PH 1, PH 2, PH 3. 1; Reversible ratchet 1/4" • material: special tool steel • surface finish: chrome plated • double component handle; 1/4" approx. length 157 mm. 1; Swivel handle 1/4" • material: chrome vanadium • double component handle • body drop forged • surface finish: chrome plated; 1/4" approx. length 150 mm. 1; Sliding T - handle 1/4" • material: chrome vanadium • surface finish: chrome plated; 1/4" approx. length 120 mm. 2; Extension bar 1/4" • material: chrome vanadium • surface finish: chrome plated; 1/4" x 55 mm, 1/4" x 150 mm. 1; Universal joint 1/4" • material: chrome vanadium • surface finish: chrome plated; 1/4". 1; Adaptor 1/4" • material: chrome vanadium • surface finish: chrome plated; 1/4" - 3/8". 1; Spinner handle 1/4" • material: blade - chrome vanadium, entirely hardened and tempered • handle - polypropylene; 1/4" approx. length 150mm. 18; Socket 1/2" • material: chrome molybdenum • surface finish: chrome plated; 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 27, 30, 32. 4; Socket with internal TX profile 1/2" • material: chrome vanadium • surface finish: chrome plated; E 10, E 11, E 12, E 14. 6; Hexagonal screwdriver socket 1/2" • material: socket: chrome vanadium, chrome plated • material: bit: special tool steel; 6, 7, 8, 9, 10, 12. 4; Screwdriver socket 1/2" with TX profile • material: socket: chrome vanadium, chrome plated • material: bit: special tool steel; TX 30, TX 40, TX 45, TX 50. 4; Long screwdriver socket 1/2" with ZX profile • material: socket: chrome vanadium, chrome plated • material: bit: special tool steel; M 6, M 8, M 10, M 12. 1; Reversible ratchet 1/2" • material: special tool steel • double component handle • surface finish: chrome plated; 1/2" approx. length 260mm. 1; Swivel handle 1/2" • material: chrome vanadium • double component handle • surface finish: chrome plated; 1/2" approx. length 380mm. 1; Sliding T - handle 1/2" • material: chrome vanadium • surface finish: chrome plated; 1/2" approx. length 300mm. 2; Extension bar 1/2" • material: chrome vanadium • surface finish: chrome plated; 1/2" x 250mm, 1/2" x 125mm. 1; Universal joint 1/2" • material: chrome vanadium • surface finish: chrome plated; 1/2". Set of screwdrivers with T handle in tool tray No. Item; Item; Dimensions of tools. 7; Ball - end hexagonal screwdriver with T - handle • material: blade - chrome vanadium • handle - polypropylene • blade chrome plated, blacken tip; 2.5 approx. length 155mm, 3 approx. length 155mm, 4 approx. length 155mm, 5 approx. length 188mm, 6 approx. length 188mm, 8 approx. length 223mm, 10 approx. length 248mm. 8; TX profile screwdriver with T-handle • material: blade - chrome vanadium, entirely hardened and tempered • handle - polypropylene • blade chrome plated, blacken tip; TX 10 approx. length 155mm, TX 15 approx. length 155mm, TX 20 approx. length 155mm, TX 25 approx. length 188mm, TX 27 approx. length 188mm, TX 30 approx. length 188mm, TX 40 approx. length 223mm, TX 45 approx. length 223mm. 1; Flat file with handle, half smooth • material: carbon tool steel • all files have second cut; 200 approx. length 325mm. 1; Three-square file with handle, half smooth • material: carbon tool steel • all files have second cut; 200 approx. length 325mm. 1; Utility knife • 8 automatic exchangeable snap-off blades inside the handle • blade from high alloy steel; 160. Set of screwdrivers in tool tray No. Item; Item; Dimensions of tools. 5; Flat screwdriver TBI • blade: chrome-vanadium-molybdenum steel • blade chrome plated, blacken tip • handle: ergonomic shape; 0.5 x 3.0 x 80 approx. total length 165mm, 0.8 x 4.0 x 100 approx. total length 185mm, 1.0 x 5.5 x 125 approx. total length 225mm, 1.2 x 6.5 x 150 approx. total length 260mm, 1.6 x 8.0 x 175 approx. total length 295mm. 4; Crosstip (PH) screwdriver TBI • blade: chrome-vanadium-molybdenum steel, • blade chrome plated, blackened tip • handle: ergonomic shape; PH 0 x 60 approx. total length145mm, PH 1 x 80 approx. total length165mm, PH 2 x 100 approx. total length 200mm, PH 3 x 150 approx. total length 260mm. 2; Stubby crosstip (PH) screwdriver TBI • blade: chrome-vanadium-molybdenum steel • blade chrome plated, blacken tip • handle: ergonomic shape; PH 1 x 25 approx. total length 87mm, PH 2 x 25 approx. total length 87mm. 2; Stubby flat screwdriver TBI • blade: chrome-vanadium-molybdenum steel • blade chrome plated, blacken tip • handle: ergonomic shape; 1.0 x 5.5 x 25 approx. total length 87mm, 1.2 x 6.5 x 25 approx. total length 87mm. 9; Screwdriver TBI with TX profile • blade: chrome-vanadium-molybdenum steel, entirely hardened and tempered • blade chrome plated, blackened tip • handle: ergonomic shape; TX 6 approx. total length 145mm, TX 8 approx. total length 145mm, TX 10 approx. total length 165mm, TX 15 approx. total length 165mm, TX 20 approx. total length 185mm, TX 25 approx. total length 200mm, TX 27 approx. total length 215mm, TX 30 approx. total length 225mm , TX 40 approx. total length 250mm. 8; Socket wrench with TBI handle • blade: chrome-vanadium-molybdenum steel • blade chrome plated • handle: ergonomic shape; 6 approx. total length 235mm, 7 approx. total length 245mm, 8 approx. total length 245mm, 9 approx. total length 245mm, 10 approx. total length 245mm, 11 approx. total length 245mm, 12 approx. total length 245mm, 13 approx. total length 245mm. Set of tools with 14 pieces in tool tray No. Item; Item; Dimensions of tools. 1; Combination pliers • material: special tool steel • cutting edges induction hardened • head polished • surface finish: chrome plated • heavy duty double - component handles; approx. length 180 mm. 1; Grip pliers • jaws drop forged from special tool steel • handles made from sheet metal • surface finish: nickel plated; approx. length 250 mm. 1; Double groove joint pliers • material: special tool steel • drop forged, entirely hardened and tempered • head polished • surface finish: chrome plated • heavy duty coated handles; approx. length 240 mm. 1; Heavy duty diagonal cutting nippers • material: special tool steel • cutting edges induction hardened • surface finish: chrome plated • heavy duty coated handles; approx. length 180 mm. 1; Long nose pliers with side cutter and pipe grip, straight • material: special tool steel • cutting edges induction hardened • surface finish: chrome plated • heavy duty coated handles • gripping surface serrated; approx. length 200 mm. 1; Long nose pliers with side cutter and pipe grip, bent • material: special tool steel • cutting edges induction hardened • surface finish: chrome plated • heavy duty double - component handles • gripping surface serrated • jaws bent to an angle of 45°; approx. length 200 mm. 1; External lock rings pliers, straight • material: special tool steel • drop forged, entirely hardened and tempered • head polished • surface finish: chrome plated • tips black finish • plastic dipped handles • for fitting circlips on shafts; approx. length 180 mm for fitting circlips on shafts 19 - 60. 1; External lock rings pliers, bent • material: special tool steel • drop forged, entirely hardened and tempered • head polished • surface finish: chrome plated • tips black finish • double plastic dipped handles • for fitting circlips on shafts; approx. length 166mm 180x 19 - 60. 1; Internal lock rings pliers, straight • material: special tool steel • surface finish: chrome plated • tips black finish • dipped handles • for fitting circlips on bore holes; approx. length 180 mm for fitting circlips on shafts 19 - 60. 1; Internal lock rings pliers, bent • material: special tool steel • head polished • surface finish: chrome plated • tips black finish • plastic dipped handles • for fitting circlips on bore holes • with inserted tips; approx. length 166 mm 180 x 19 - 60. 1; Locksmiths` hammer • material: special tool steel • drop forged • striking face and nose induction hardened • surface finish: lacquered • ash wood handle with special oval wedge; Impact had approx. 400 gr wood handle approx. length 310mm. 1; Centre punch • material: chrome vanadium • tip induction hardened • surface finish: lacquered; 5- length of tool approx. 120mm. 1; Flat chisel • material: chrome vanadium • drop forged • tip induction hardened • surface finish: lacquered; Approx. length 200 mm Tip 24x3. 1; Bumping hammer • material: celidor • oil resistant • gray cast head • wooden handle, lacquered; Approx. length 320mm Impact had 40. Warranty minimum 1 year Equal or similar to the model UNIOR TEOS EUROVISION 940EV3 | 10 | ☐ Yes ☐ No |  |

Delivery requirements –– Comparative Data Table

| UNOPS Requirements |  | Is quotation compliant? Bidder to complete | Details Bidder to complete Insert details |
| --- | --- | --- | --- |
| Delivery time | Bidder shall deliver the goods as soon as possible but latest within 30 (thirty) calendar days after Contract signature. | ☐ Yes ☐ No |  |
| Delivery place and Incoterms rules | DAP (Delivery at Place), offload included, as per Incoterms 2010, net of any direct taxes, customs duties and indirect taxes and VAT. | ☐ Yes ☐ No |  |
| Consignee details | The goods should be delivered and offloaded on one address in Belgrade. | ☐ Yes ☐ No |  |
| UNOPS Right to vary requirements | At the time the Contract is awarded, UNOPS reserves the right to vary the quantity of the goods and associated services specified above, provided this does not exceed +/- 30%, without any change in the unit prices or other terms and conditions of the RFQ. | ☐ Yes ☐ No |  |

The offered goods and related services (if applicable) are in accordance with the required specifications and requirements specified in Section III: Schedule of Requirements.

☐ Yes   ☐ No

ANY DEVIATION MUST BE LISTED BELOW:

_________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

Name	: _____________________________________________________________

Title	: _____________________________________________________________

Date	: _____________________________________________________________

Signature	: _____________________________________________________________

Form D: Previous experience form

RFQ reference no: [insert RFQ reference No.]_________________________________

Name of Bidder: [insert name of Bidder]______________________________________

| Description of services/goods | Country | Total amount of Contract | Contract Identification and Title and Contact details of Client: (Name, Address, telephone, email, fax) | Year project was undertaken |
| --- | --- | --- | --- | --- |

Name	: _____________________________________________________________

Title	: _____________________________________________________________

Date	: _____________________________________________________________

Signature	: _____________________________________________________________75 00ax: +45 45 33 75 01